Guide for DeepL SR-SMLM

by Hanna MANKO

11.10.2021

- Contents

# Preparation of the patch sets……………..………..………..………..………..………..………..………..………..………..….….1

# Training the Neural Network……………..………..………..………..………..………..………..………..………..………..….…..1

Introduction

This is a guide that shows how to prepare the training set and train a network for SR-SMLM. The training set contains corresponding low and high SNR images creates from same data.

- Preparation of patch sets

- #####################  Import the required libraries  ###################

- import numpy as np

- import pandas as pd

- from tifffile import imread, imsave

- from sklearn.cluster import DBSCAN

- from matplotlib import pyplot as plt

- from numpy import unique

- from numpy import where

- import time

- import math

- import random

- from skimage.morphology import disk

- from scipy.ndimage.morphology import white_tophat

- from joblib import Parallel, delayed

- ### Defining Patches Size

- image_height = 16  ## image height in pixels

- loc_image_width = 16  ## image width in pixels

- image_chanels = 1   ##

- loc_sp_distance = 230  ## distance between localization and its spectral peak

- #   measured from original image / in pixels

- sp_image_width = 48 #### spectral image width

- half_width = int(loc_image_width/2)    ##

Next we need to define all the functions we will use later

- ﻿##################### DEFINING THE  FUNCTIONS ###################

- ###

- ##### #######    Useful  function to plot on one figure multiple patches

- def show_images(patches):

- n = int(math.sqrt(len(patches))+2)

- m = int(math.sqrt(len(patches)))

- plt.figure(figsize = (50,40))

- for i in range (0, len(patches)):

- plt.subplot(n,m,i+1)

- plt.title(i, fontsize=25)

- plt.imshow(patches[i])

- plt.show()

- #######

- def set_coordinates(i, half_width, loc_sp_distance, sp_image_width):

- row = np.array(where(cluster == i))

- ## coordinates for localization part

- shift_y = random.randint(-6,6)   ##    shift for x and y coordinates. To move the box around localization

- shift_x = random.randint(-6,6)   ##

- # shift_y = 0   ##      if shift is not required. When we need to create patches

- # shift_x = 0   ##

- y1 = int(dat[np.array(row)[0,0],0]-half_width-shift_y)   #   start y-coordinate of the box around localization

- y2 = int(dat[np.array(row)[0,0],0]+half_width-shift_y)   #   end y-coordinate of the box around localization

- x1 = int(dat[np.array(row)[0,0],1]-half_width-shift_x)   #   start x-coordinate of the box around localization

- x2 = int(dat[np.array(row)[0,0],1]+half_width-shift_x)   #   end y-coordinate of the box around localization

- ## coordinates for spectral part

- y11 = int(dat[np.array(row)[0,0],0]+loc_sp_distance-shift_y)    #   start y-coordinate of the box around spectra

- y21 = int(dat[np.array(row)[0,0],0]+loc_sp_distance + sp_image_width-shift_y) #   end y-coordinate of the box around spectra

- return(y1,y2,x1,x2,y11,y21,row)

- ####   The code was first oncate to be able to create patches of different types

- ##   Function that returns image shape depending on patch type chosen

- def image_type(image_height, sp_image_width,loc_image_width):

- if only_loc == True:                                       ##  to have only localization part

- im_shape = (image_height, loc_image_width)

- if only_spectra == True:                                   ##  to have only spectral part

- im_shape = (image_height, sp_image_width)

- if combined == True:                                       ##  to have both localization and spectral parts

- im_shape = (image_height, sp_image_width+loc_image_width)

- return (im_shape)

- #   Function that forms the patches depending on patch type chosen

- sp_cor = 0   ###  the number of pixels in x-axis to correct position of the box around spectral part

- def patches_formation(row, stack):

- #sp_cor = random.randint(-2,8)  ##    to introduce random shift for the box around spectral part

- frame = dat[:,2][row]

- if only_loc == True:

- patch = np.float64(stack[frame, x1:x2, y1:y2])

- patch = patch.reshape(row.shape[1],16,16)

- if only_spectra == True:

- patch = np.float64(stack[frame, x1:x2, y11:y21])

- patch = patch.reshape(row.shape[1],16,48)

- if combined == True:

- patch = np.concatenate((np.float64(stack[frame, x1:x2, y1:y2]),

- np.float64(stack[frame, x1-sp_cor:x2-sp_cor, y11:y21])), axis=3)

- patch = patch.reshape(row.shape[1],16,64)

- return(patch)

- #######   Function to calculate Spatial Frecuency as described in (Shutao Li et al.

- # ‘Combination of images with diverse focuses using the spatial frequency’,  Information Fusion

- # https://doi.org/10.1016/S1566-2535(01)00038-0.

- def SF_calculator(patch):

- MN = image_height*sp_image_width

- rf = np.sqrt((1/MN)*np.sum([np.abs((patch[:,n]-patch[:,n-1])**2) for n in range(1,sp_image_width-1)]))

- cf = np.sqrt((1/MN)*np.sum([np.abs((patch[m,:]-patch[m-1,:])**2) for m in range(1,image_height-1)]))

- SF = np.sqrt(rf**2 + cf**2)

- return SF

- # Two functions to denoise spectral and localization  part

- selem = disk(10)

- def d_spectra(img, tempIm, ampFact = .5):   ## ampFact is introduced to increase signal for the spectral part

- imgMean1 = tempIm[0, 0:img.shape[2]]/(ampFact*tempIm[0:img.shape[1],0:img.shape[2]].max())

- imgMean1 = white_tophat(imgMean1, footprint =  selem)

- return(imgMean1)

- def d_loc(img, tempIm):

- imgMean2 = tempIm[0, 0:img.shape[2]]/tempIm[0:img.shape[1],0:img.shape[2]].max()

- imgMean2 = white_tophat(imgMean2, footprint = selem)

- return(imgMean2)

- #######    /// Spatial Frequency calculation and image oncatena  \\\

- def SF_Image(patch, only_loc=False, only_spectra=False, combined=False, denoising = True):

- SF=[]

- SF_sum = 0

- cumIm = np.zeros((patch[1:].shape))

- for I in range(0,len(patch[1:])):

- SF.append(SF_calculator(patch[i]))

- cumIm = cumIm + patch[i]*SF[i]

- SF_sum = np.sum(SF)

- tempIm = cumIm/SF_sum

- if denoising ==True:

- if only_spectra == True:

- imgMean = d_spectra(patch[:,:,loc_image_width:], tempIm[:,:,loc_image_width:])

- elif only_loc == True:

- imgMean = d_loc(patch[:,:,0:loc_image_width], tempIm[:,:,0:loc_image_width])

- elif combined == True:

- imgMean = np.concatenate((d_loc(patch[:,:,0:loc_image_width],

- tempIm[:,:,0:loc_image_width]),(d_spectra(patch[:,:,loc_image_width:],

- tempIm[:,:,loc_image_width:]))),axis=1)

- return(imgMean)

- else:

- return(tempIm)

- ####

- #   function that exclude one frame and apply SF_Image function to all othe frames

- def SF_part(n, patch_x, pat_s):

- fr = [*range(0,len(patch_x))]

- fr.pop(n)

- pat_s = SF_Image(patch_x[((fr)),:,:], combined=True)

- return (pat_s)   #

Data preparation part:

- ﻿############################################################### 3

- ############################################################### 2

- ############################################################### 1 ...

- ##########################   ## Put the type of images that you want to obtain equal True

- ###///                          Here we will create images containing both spectral and localization parts

- only_spectra = False

- only_loc = False

- combined = True

- ###\\\

- ##########################

- # Reading the oncatenate file created in one of the next plugins (line 147, line 158)

- data = pd.read_csv(‘/Users/hannamanko/Desktop/diff_st/My Library_2/U-net/for figures/50mW_4_THSt_5.0.csv’)

- ####    localization file from ThunderSTORM (Fiji plugin)  ###

- # ////

- data = data[data[“Intensity [photon]”]>10000]  ## you can filter the data by intensity to discard noise localizations

- markerSize = 0.05

- plt.plot(data[“x [nm]”], data[“y [nm]” ], ‘o’, markersize=markerSize, color=’red’)

- datat = pd.concat([pd.DataFrame(data[“x [nm]”]),pd.DataFrame(data[“y [nm]”]),pd.DataFrame(data[“frame”])], axis = 1, ignore_index=False)

- datat[‘y [nm]’] = (datat[‘y [nm]’]/21.36752)/5

- datat[‘x [nm]’] = (datat[‘x [nm]’]/21.36752)/5

- ### \\\\

- ####    localization file from PeakFit (part of GDSC SMLM2 plugin in Fiji)  ###

- # /////

- data = data[data[“Signal”]>35000]

Filtered data

Unfiltered data

plt.plot(data[“origX”], data[“origY” ], ‘o’, markersize=markerSize, color=’red’)

- datat = pd.concat([pd.DataFrame(data[“origX”]),pd.DataFrame(data[“origY”]),pd.DataFrame(data[“Frame”])], axis = 1, ignore_index=False)

- data_n = datat.to_numpy()

- ### \\\\\

- data_n= datat.to_numpy()

- d = data_n[(data_n[:,0]>8)&(data_n[:,0]<220)&(data_n[:,1]>8)&(data_n[:,1]<200)]

- plt.plot(d[:,0], d[:,1], ‘o’, markersize=markerSize, color=’red’)

- #####

- ##############################################################################

- #####       Next we cluster the localizations on consecutive frames to create ‘clean’ images from the same localization

- ##          For this we are using DBSCAN

- cl = DBSCAN(eps=0.5,min_samples=3)    # clustering   ## eps need to be choosed depending on the data (see line 203)

- cluster = cl.fit_predict(d[:, 0:2])   # table with indexes of all clusters

- clusters = unique(cluster)            # tacking unique indexes of clusters

- clusters = clusters[clusters>-1]      # discarding noise

- #---

- def consecutive(data, stepsize=1):             # small function that helps to find consecutive frames for cluster

- return np.split(data, np.where(np.diff(data) != stepsize)[0]+1)

- #---

- dat = pd.concat([pd.DataFrame(d), pd.DataFrame(cluster)], axis = 1)  # Adding cluster indexes as additional column to our data

- dat = dat.to_numpy()  # Converting to numpy

- cluster_2 = np.int64(np.full((cluster.shape), -1))  # Creation array full of ‘-1’

- new_cluster = 0

- for I in clusters:

- ar = consecutive(dat[:,2][(where(dat[:,3] == i))])  # I consecutive indexes in data for each cluster

- for j in ar:             # for ezch of the consecutive index sets

- if len(j) > 8:       # if this set is longer then 8

- for ii in range(0, len(j)):

- cluster_2[where((dat[:,2]==j[ii])&(dat[:,3] == i))] = int(new_cluster)  # put new cluster index to cluster_2

- print(new_cluster)

- new_cluster = new_cluster+1  # increase new_cluster index by 1

- clusters_2 = unique(cluster_2)  #  again finding unique clusters

- clusters_2 = clusters_2[clusters_2>-1]  # discarding the noise

- dat[:,3] = cluster_2   #  put cluster indexes as column 3 in data

- clusters = clusters_2  #  rewrite clusters

- del cluster_2, clusters_2  # deleting additional variables

- for clust in clusters:         ##  Using this loop we can plot data by coloring different clusters (helps to choose eps)

- row_ix = where(cluster == clust)

- plt.scatter(dat[row_ix, 0], dat[row_ix, 1], s=markerSize)

Good eps

Example of too big eps

plt.show() ##

Here is the main part of code that creates patch sets

- #  Reading the original stack of acquired images

- stack = imread(“/Users/hannamanko/Desktop/diff_st/My Library_2/U-net/for figures/50mW_4_MMStack_Pos0.ome-1.tif”)

- ﻿start_time = time.time()

- print(“--- %s seconds ---" ,(time.time() – start_time))   # printing start time

- ### Creating the training set

- ##########

- im_shape = image_type(image_height, sp_image_width,loc_image_width)   # Reading image shape

- count = 0

- print(“Please, wait until the end”)

- start_time = time.time()

- for I in clusters:      #  for each cluster index

- y1,y2,x1,x2,y11,y21,row = set_coordinates(I,half_width,loc_sp_distance, sp_image_width)  #  defining the coordinates of the box

- patch_x = np.zeros((np.shape(row[0])[0], im_shape[0], im_shape[1]))  # creating the empty patch of defined size

- try:

- patch = patches_formation(row, stack)  #  forming the patches for one cluster (consecutive localizations)

- except:

- patch.shape != im_shape

- patch_x = patch

- pat_s = np.zeros(patch_x.shape)

- SF_p = []

- SF_sum = []

- pat_s = Parallel(n_jobs=5)(delayed(SF_part)(n, patch_x, pat_s) for n in range(0, len(patch_x)))  # creating ‘clean’ images

- pat_s = np.array(pat_s).reshape(patch_x.shape)  # reshaping

- SF_sum = [SF_calculator(pat_s[n]) for n in range(0, len(pat_s))]   # saving calculated SF value

- SF_p = [SF_calculator(patch_x[n]) for n in range(0, len(patch_x))]  # calculating SF value for one excluded patch

- ind = where(np.array(SF_sum)/np.array(SF_p) > ((np.array(SF_sum)/np.array(SF_p)).max()-    #

- (np.array(SF_sum)/np.array(SF_p)).min())/2) #

- patch_ = patch_x[ind]

- patch_sum  = pat_s[ind]

- if count == 0:                                                     #   gathering all the patches in one big set

- patch__ = patch_                                               #   set of original images

- patch_sum_  = patch_sum                                        #   set of created ‘clean’ images

- else:                                                              #

- patch__ = np.concatenate((patch__, patch_), axis=0)            #   set of original images

- patch_sum_ = np.concatenate((patch_sum_, patch_sum), axis=0)   #   set of created ‘clean’ images

- count = count+1

- print(count)

- print(“--- %s seconds ---" % (time.time() – start_time))    #  printing  end time

- #  Now we can save created raw and ‘clean’ sets of patches

- imsave(‘path/patch_sum.tif’, patch_sum_)

- imsave(‘path/patch.tif’, patch__)

In [9]: ….

--- %s seconds --- 0.0

Please, wait until the end

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

...

This process can take a significant amount of time so it is better to use more powerful computer

In [10]: show_images(patch__[60:140])  ## building the figure with some of the patches with raw images

In [11]: show_images(patch__[60:140])  ## building the figure with some of the patches with created ‘clean’ images

this part is to create noisy patches which are required to properly train the network

- ﻿#################################

- #########   to create noisy patches

- #################################

- #  We can take any of the stacks used to create patches for the training set

- stack = imread(‘D:/1/Data/GattaQuant/Gqaunt_ Fluoro150mW_sPAINT__11/Gqaunt_ Fluoro150mW_sPAINT__11_MMStack_Pos0.ome.tif’)

- stack = stack/60000

- #########    To create big set of noisy patches this process need to be repeated several times

- #########    and obtained sets need to be concatenated

- ###   :::::::::

- im_shape = image_type(image_height, sp_image_width,loc_image_width)

- count = 0

- for I in range(0, len(stack)):

- for _ in range(0, 40):

- rand_x = random.randint(13,215)

- rand_y = random.randint(10,220)

- y1 = rand_y-half_width

- y2 = rand_y+half_width

- x1 = rand_x-half_width

- x2 = rand_x+half_width

- ## coordinates for spectral part

- y11 = rand_y+loc_sp_distance

- y21 = rand_y+loc_sp_distance + sp_image_height

- patch = np.concatenate((np.float64(stack[I, x1:x2, y1:y2]),

- np.float64(stack[I, x1:x2, y11:y21])), axis=1)

- try:

- patch = patch

- except:

- patch.shape != im_shape

- pat_s = SF_Image(np.concatenate((patch.reshape(1, 16, 64), patch.reshape(1, 16, 64)), axis = 0),combined=True)

- patch_ = np.concatenate((patch.reshape(1, 16, 64), patch.reshape(1, 16, 64)), axis = 0)

- patch_sum  = pat_s

- if count == 0:

- patch__ = patch_[0].reshape(1,16,64)

- patch_sum_  = patch_sum.reshape(1,16,64)

- else:

- patch__ = np.concatenate((patch__, patch_[0].reshape(1,16,64)), axis=0)

- patch_sum_ = np.concatenate((patch_sum_, patch_sum.reshape(1,16,64)), axis=0)

- count = count+1

- print(count)

- patch__ = imread(‘D:/My Library_2/U-net/For_Network/xx/Noise3.tif’)

- patch_sum_ = imread(‘D:/My Library_2/U-net/For_Network/yy/Noise3.tif’)

- #  Filtering and deleting the patches that randomly got localization

- i_list = []

- for I in range(0, len(patch_sum_)):

- if patch_sum_[I,:,:16].max() > 0.3:

- i_list.append(i)

- patch_sum_ = np.delete(patch_sum_, [i_list], axis=0)

- patch__ = np.delete(patch__, [i_list], axis=0)

In [12]: show_images(patch__[60:140])  ## building the figure with some of the ‘noisy’ patches with ‘raw’ images

In [13]: show_images(patch_sum_[60:140])  ## building the figure with some of the ‘noisy’ patches with created ‘clean’ images

- ###    To concatenate  several sets of patches:

- p = patch__   #  writing set of raw patches to new variable

- ps = patch_sum_  #  writing set of ‘clean’ patches to new variable

- ## Sometimes to obtain required number of images in patch set

- patch__ = np.concatenate((patch__, p), axis = 0)      #  gathering all the patch sets

- patch_sum_ = np.concatenate((patch_sum, ps), axis=0)

- ###   :::::::::

- #########

- ﻿##############################

This part usually used after we have a trained network to treat new data

- ##############################

- ###  Creating the same type of patches but only the ‘raw’, so basically we just cut the images in the correct form.

- ###  These patches are created to be treated with our model and get images with high SNR at the output

- #

- #######################       !!!!  In this part the shift of the box is not used anymore,

- #  so all the variables that induce

- #  shift in the ‘Defining functions’ part need to be disabled

- ###      +   for this part the data should not be clustered so we use array d created in line 167   !!!!   ########

- ##############################

- x1_list = []  #  list to save original x-coordinates

- y1_list = []  #  list to save original y-coordinates

- count = 0

- for I in range(0,len(d)):

- y1 = int(d[i][:1][0])-half_width

- y2 = y1+2*half_width

- x1 = int(d[i][1:2][0])-half_width

- x2 = x1+2*half_width

- ## coordinates for spectral part

- y11 = int(d[i][:1][0])+loc_sp_distance

- y21 = int(d[i][:1][0])+loc_sp_distance + sp_image_width

- try:

- patch = np.concatenate(((stack[int(d[i][2]), x1:x2, y1:y2]),

- (stack[int(d[i][2]), x1:x2, y11:y21])), axis=1)

- if (patch[8,1]+patch[8,8]+patch[8,15]) == 0:

- continue

- y1_list.append(d[i][1:2][0])

- x1_list.append(d[i][:1][0])

- patch = patch.reshape(1,16,64)

- except:

- continue

- if count == 0:

- patch_x = patch

- else:

- patch_x = np.concatenate((patch_x, patch), axis=0)

- count = count+1

- print(count)

- coordinates = np.zeros((len(x1_list),2))   # creating array

- coordinates[:,0]= x1_list                  # adding x coordinates to 0 column

- coordinates[:,1]= y1_list                  # adding y coordinates to 1st column

- plt.scatter(coordinates[:,0],coordinates[:,1],s=markerSize,cmap=plt.cm.jet) # we can plot the coordinates to check if everything is ok

In [14]: show_images(patch_x[140:200])  ## building the figure with some of the patches ()

- #  Saving the original coordinates as *.csv file and created patch as *.tif

- pd.DataFrame(coordinates).to_csv(‘/Users/hannamanko/Desktop/diff_st/My Library_2/U-net/for figures/coordinates_5.0.csv’,index = False)

- imsave(‘/Users/hannamanko/Desktop/diff_st/My Library_2/U-net/for figures/50mW_4_patch_m5.0.tif’, patch_x)

- ##############################

Small additional part to draw rectangles around localizations and corresponding spectra

- ﻿###################################################

- #################     To draw rectangles

- ###################################################

- from matplotlib.patches import Rectangle

- dd = stack[980]  # Choosing one frame from the images stack

- plt.imshow(dd)   # showing this image

- datt = data.iloc[where(data[‘Frame’] == 980)]  # choosing localizations only from this frame

- datt = datt[datt[‘origX’]<250]

- datt = datt[datt[‘Signal’]>60000]    # Filtering by intensity

- cord = pd.concat((datt[‘origX’], datt[‘origY’]), axis = 1, ignore_index=False)plt.scatter(loc[‘origX’], loc[‘origY’])

- ax = plt.gca()

- rect = Rectangle((x1,y1),16,16,linewidth=1,edgecolor=’r’,facecolor=’none’) # Creating a Rectangle

- ax.add_patch(rect)

- for I in range(0, len(cord)):

- y1 = cord.iloc[I,1]-half_width

- x1 = cord.iloc[I,0]-half_width

- y11 = cord.iloc[I,0]+loc_sp_distance

- plt.imshow(dd)

- plt.gca().add_patch(Rectangle((x1,y1),16,16,linewidth=1,edgecolor=’r’,facecolor=’none’))

- plt.gca().add_patch(Rectangle((y11,y1),48,16,linewidth=1,edgecolor=’green’,facecolor=’none’))

- plt.show()

All the created images can be saved as .tif stacks then it is possible to open it using Fiji if it is required. The other possibility is to save it as *.csv table, this way it is faster to upload the images in python.

Here you can see first way, when stacks were created separately so it required to concatenate everything into one big stack:

- ﻿##############################################

- ########      Downloading image stacks    ####

- X_train_25_2 = imread('D:/My Library_2/U-net/For_Network/x_new/25_2_patch_tr.tif')

- X_train_50_1 = imread('D:/My Library_2/U-net/For_Network/x_new/50_1_patch_tr.tif')

- X_train_50_2 = imread('D:/My Library_2/U-net/For_Network/x_new/50_2_patch_tr.tif')

- X_train_75_1 = imread('D:/My Library_2/U-net/For_Network/x_new/75_1_patch_tr.tif')

- X_train_75_2 = imread('D:/My Library_2/U-net/For_Network/x_new/75_2_patch_tr.tif')

- X_train_100_1 = imread('D:/My Library_2/U-net/For_Network/x_new/100_1_patch_tr.tif')

- X_train_100_2 = imread('D:/My Library_2/U-net/For_Network/x_new/100_2_patch_tr.tif')

- X_train_125_2 = imread('D:/My Library_2/U-net/For_Network/x_new/125_2_patch_tr.tif')

- X_train_125_3 = imread('D:/My Library_2/U-net/For_Network/x_new/125_3_patch_tr.tif')

- X_train_150_1 = imread('D:/My Library_2/U-net/For_Network/x_new/150_1_patch_tr.tif')

- X_train_150_2 = imread('D:/My Library_2/U-net/For_Network/x_new/150_2_patch_tr.tif')

- X_noise = imread('D:/My Library_2/U-net/For_Network/xx/Noise.tif')

- X_noise2 = imread('D:/My Library_2/U-net/For_Network/xx/Noise2.tif')

- X_noise3 = imread('D:/My Library_2/U-net/For_Network/xx/Noise3.tif')

- X_train = np.concatenate((X_noise/1.5, X_train_25_2,X_train_50_1,X_train_50_2,

- X_train_75_1,X_train_75_2,X_train_100_1,

- X_train_125_2,X_noise2/1.5,X_train_125_3,X_train_150_1,X_train_150_2,X_noise3/1.5 ), axis=0)

- del X_train_25_2,X_train_50_1,X_train_50_2

- del X_train_75_1,X_train_75_2,X_train_100_1,X_train_100_2

- del X_train_125_2,X_train_125_3,X_train_150_1,X_train_150_2, X_noise, X_noise2, X_noise3

- X_train = X_train/60000  # normalisation

- X_train = X_train.reshape((-1, image_height, image_width, 1))

- ###//////////////

- Y_train_25_2 = imread('D:/My Library_2/U-net/For_Network/y_new/25_2_patch_sum.tif')

- Y_train_50_1 = imread('D:/My Library_2/U-net/For_Network/y_new/50_1_patch_sum.tif')

- Y_train_50_2 = imread('D:/My Library_2/U-net/For_Network/y_new/50_2_patch_sum.tif')

- Y_train_75_1 = imread('D:/My Library_2/U-net/For_Network/y_new/75_1_patch_sum.tif')

- Y_train_75_2 = imread('D:/My Library_2/U-net/For_Network/y_new/75_2_patch_sum.tif')

- Y_train_100_1 = imread('D:/My Library_2/U-net/For_Network/y_new/100_1_patch_sum.tif')

- Y_train_100_2 = imread('D:/My Library_2/U-net/For_Network/y_new/100_2_patch_sum.tif')

- Y_train_125_2 = imread('D:/My Library_2/U-net/For_Network/y_new/125_2_patch_sum.tif')

- Y_train_125_3 = imread('D:/My Library_2/U-net/For_Network/y_new/125_3_patch_sum.tif')

- Y_train_150_1 = imread('D:/My Library_2/U-net/For_Network/y_new/150_1_patch_sum.tif')

- Y_train_150_2 = imread('D:/My Library_2/U-net/For_Network/y_new/150_2_patch_sum.tif')

- Y_noise = imread('D:/My Library_2/U-net/For_Network/yy/Noise.tif')

- Y_noise2 = imread('D:/My Library_2/U-net/For_Network/yy/Noise2.tif')

- Y_noise3 = imread('D:/My Library_2/U-net/For_Network/yy/Noise3.tif')

- Y_train = np.concatenate((Y_noise/1.5, Y_train_25_2, Y_train_50_1,Y_train_50_2,

- Y_train_75_1,Y_train_75_2,Y_train_100_1,

- Y_train_125_2,Y_noise2/1.5,Y_train_125_3,Y_train_150_1, Y_train_150_2, Y_noise3/1.5), axis=0)

- del Y_train_25_2,Y_train_50_1,Y_train_50_2

- del Y_train_75_1,Y_train_75_2,Y_train_100_1,Y_train_100_2

- del Y_train_125_2,Y_train_125_3,Y_train_150_1,Y_train_150_2, Y_noise, Y_noise2, Y_noise3

- Y_train = Y_train/Y_train.max()

- Y_train = Y_train.reshape(-1, image_height, image_width, 1)

- ﻿###. And we can save final stacks for training set

- imsave('C:/Users/hmanko/Desktop/X_train.tif', X_train)

- imsave('C:/Users/hmanko/Desktop/Y_train.tif', Y_train)

- Training the Neural Network

- ﻿import keras

- import tensorflow as tf

- import os

- import numpy as np

- from matplotlib import pyplot as plt

- import random

- from tifffile import imread, imsave

- from PIL import Image

- from skimage import io, img_as_ubyte

- from skimage.transform import resize, rescale

- import random

- import pandas as pd

- from tqdm import tqdm

- from tensorflow.keras.layers import Lambda,Input,Conv2D,BatchNormalization,AveragePooling2D,LeakyReLU,Conv2DTranspose,concatenate,UpSampling2D,Dropout

- from skimage.morphology import disk, white_tophat

- from numpy import expand_dims

- from keras.preprocessing.image import ImageDataGenerator

- image_width = 64

- image_height = 16

- image_chanels = 1

- ﻿

- X_train =  imread('C:/Users/hmanko/Desktop/X_train.tif')

- Y_train = imread('C:/Users/hmanko/Desktop/Y_train.tif')

- inputs = Input((image_height, image_width, image_chanels))

- c0=Conv2D(16, (6,6),activation = 'elu',kernel_initializer='he_normal', padding = 'same')(inputs)

- c0=BatchNormalization(axis=-1)(c0)

- c0=LeakyReLU(alpha=0.2)(c0)

- c0=Conv2D(16, (6,6),activation = 'elu',strides = 2,kernel_initializer='he_normal', padding = 'same')(c0)

- c0=BatchNormalization(axis=-1)(c0)

- c0=LeakyReLU(alpha=0.2)(c0)

- c1=Conv2D(32, (6,6),activation = 'elu',kernel_initializer='he_normal', padding = 'same')(c0)

- c1=BatchNormalization(axis=-1)(c1)

- c1=LeakyReLU(alpha=0.2)(c1)

- c1=Conv2D(32, (6,6),activation = 'elu',strides = 2, kernel_initializer='he_normal', padding = 'same')(c1)

- c1=BatchNormalization(axis=-1)(c1)

- c1=LeakyReLU(alpha=0.2)(c1)

- c2=Conv2D(64, (6,6),activation = 'elu',kernel_initializer='he_normal', padding = 'same')(c1)

- c2=BatchNormalization(axis=-1)(c2)

- c2=LeakyReLU(alpha=0.2)(c2)

- c2=Conv2D(64, (6,6),activation = 'elu',strides = 2,kernel_initializer='he_normal',padding = 'same')(c2)

- c2=BatchNormalization(axis=-1)(c2)

- c2=LeakyReLU(alpha=0.2)(c2)

- c3=Conv2D(128, (6,6),activation = 'elu',kernel_initializer='he_normal', padding = 'same')(c2)

- c3=BatchNormalization(axis=-1)(c3)

- c3=LeakyReLU(alpha=0.3)(c3)

- u4=Conv2DTranspose(64, (6,6), padding ='same')(c3)

- u4=concatenate([u4,c2])

- c4=UpSampling2D(size=2)(u4)

- c4=Conv2D(64, (6,6),activation = 'elu',kernel_initializer='he_normal', padding = 'same')(c4)

- c4=BatchNormalization(axis=-1)(c4)

- c4=LeakyReLU(alpha=0.2)(c4)

- u5=Conv2DTranspose(32, (6,6), padding ='same')(c4)

- u5=concatenate([u5,c1])

- c5=UpSampling2D(size=2)(u5)

- c5=Conv2D(32, (6,6),activation = 'elu',kernel_initializer='he_normal', padding = 'same')(c5)

- c5=BatchNormalization(axis=-1)(c5)

- c5=LeakyReLU(alpha=0.2)(c5)

- u6=Conv2DTranspose(16, (6,6), padding ='same')(c5)

- u6=concatenate([u6,c0])

- c6=UpSampling2D(size=2)(u6)

- c6=Conv2D(16, (6,6),activation = 'elu',kernel_initializer='he_normal', padding = 'same')(c6)

- c6=BatchNormalization(axis=-1)(c6)

- c6=LeakyReLU(alpha=0.2)(c6)

- outputs = Conv2D(1, (1,1), activation ='relu')(c6)

- model = tf.keras.Model(inputs = [inputs], outputs = [outputs])

- model.compile(optimizer = 'rmsprop', loss="mean_squared_error")

- model.summary()

- reduce_lr = tf.keras.callbacks.ReduceLROnPlateau(monitor='val_loss', factor=0.2,patience=4, min_lr=0.001)

- earlyStop = keras.callbacks.EarlyStopping(patience=10, verbose=1, restore_best_weights=True)

- callbacks_list = [earlyStop, reduce_lr]

- history = model.fit(X_train,Y_train,validation_split=0.1, batch_size=80, epochs=40)

- model.save("D:/My Library_2/model_mix_30.07_19_BS_50(6,6)_.h5")  # saving the model

- ﻿

- #############################################

- ##    Downloading the test patches

- X_test = imread('/path/*.tif')

- X_test = X_test/60000

- X_test = X_test.reshape(-1, image_height, image_width, 1)

- Y_test = imread('/path/*.tif')

- Y_test = Y_test/60000

- Y_test = Y_test.reshape(-1, image_height, image_width, 1)

- prediction = model.predict(X_test)

- ######################

- num = random.randint(1,len(X_test))

- plt.figure(figsize = (25,10))

- plt.subplot(221)

- plt.plot(list(range(0,64)),X_test[num,10,:,0])

- plt.plot(list(range(0,64)),Y_test[num, 8,:,0])

- plt.plot(list(range(0,64)),prediction[num, 10,:,0])

- plt.legend(['X_pos', 'Y_pos','prediction'])

- plt.subplot(222)

- plt.imshow(prediction[num,:,:,0])

- plt.title("Prediction")

- plt.subplot(223)

- plt.imshow(X_test[num,:,:,0])

- plt.title("Test example")

- plt.subplot(224)

- plt.imshow(Y_test[num,:,:,0])

- plt.title("Clean image")

Here, because ‘clean’ image is created using multiple images of the same localization we can see other localizations appearing on the image, that do not correspond to raw images.

plt.show()

- imsave('D:/1/Data/GataQuand__15-150mW/50_4_pred.tif', prediction)

- ﻿

- ##  To download already trained model

- model = load_model("D:/My Library_2/model_mix_19.08_25_BS_80(6,6)_.h5")